მოსამზადებელი ტოპოლოგია.

ამოცანის პირობა რის მიხედვითაც აიგო:

ზუსტი ინსტრუქცია:

STEP 1 — Router0 (CENTER) interfaces (IP + serial):

enable

conf t

interface g0/2

ip address 10.10.10.1 255.255.255.224

no shutdown

exit

interface g0/0

ip address 12.1.0.1 255.255.255.252

no shutdown

exit

interface s0/3/0

ip address 12.2.0.2 255.255.255.252

no shutdown

exit

interface g0/1

no shutdown

exit

interface g0/1.20

encapsulation dot1Q 20

ip address 10.10.20.1 255.255.255.0

exit

interface g0/1.30

encapsulation dot1Q 30

ip address 10.10.30.1 255.255.255.0

exit

end

wr

STEP 2 — Router2 (TOP) interfaces + OSPF:

enable

conf t

interface g0/0

ip address 10.10.40.1 255.255.255.0

no shutdown

exit

interface s0/3/0

ip address 12.2.0.1 255.255.255.252

no shutdown

exit

router ospf 1

network 12.2.0.0 0.0.0.3 area 0

network 10.10.40.0 0.0.0.255 area 0

end

wr

STEP 3 — Router1 (RIGHT) interfaces + DHCP + RIP:

enable

conf t

interface g0/0

ip address 12.1.0.2 255.255.255.252

no shutdown

exit

interface g0/1

ip address 12.4.0.1 255.255.255.0

no shutdown

exit

ip dhcp excluded-address 12.4.0.1 12.4.0.20

ip dhcp pool DHCP_RIGHT

network 12.4.0.0 255.255.255.0

default-router 12.4.0.1

dns-server 8.8.8.8

exit

router rip

version 2

no auto-summary

network 12.1.0.0

network 12.4.0.0

passive-interface g0/1

end

wr

STEP 4 — Switch2 + Switch3 VLAN + trunk (Inter-VLAN part):

Switch2 (BOTTOM LEFT):

enable

conf t

vlan 20

vlan 30

exit

interface fa0/1

switchport mode trunk

switchport trunk allowed vlan 20,30

exit

interface fa0/2

switchport mode trunk

switchport trunk allowed vlan 20,30

exit

interface range fa0/3 - 5

switchport mode access

switchport access vlan 20

spanning-tree portfast

exit

end

wr

Switch3 (BOTTOM RIGHT):

enable

conf t

vlan 20

vlan 30

exit

interface fa0/4

switchport mode trunk

switchport trunk allowed vlan 20,30

exit

interface range fa0/1 - 3

switchport mode access

switchport access vlan 30

spanning-tree portfast

exit

end

wr

STEP 5 — Router0: Routing (OSPF + RIP + redistribution):

enable

conf t

router ospf 1

passive-interface default

no passive-interface s0/3/0

network 12.2.0.0 0.0.0.3 area 0

network 10.10.10.0 0.0.0.31 area 0

network 10.10.20.0 0.0.0.255 area 0

network 10.10.30.0 0.0.0.255 area 0

redistribute rip subnets

exit

router rip

version 2

no auto-summary

passive-interface default

no passive-interface g0/0

network 12.1.0.0

network 10.10.10.0

network 10.10.20.0

network 10.10.30.0

redistribute ospf 1 metric 2

exit

end

wr

STEP 6 — PC IP settings (exactly what to type)

Left Static LAN (Switch4) — 10.10.10.0/27

Gateway: 10.10.10.1
Mask: 255.255.255.224

- PC10: 10.10.10.10

- PC11: 10.10.10.11

- PC12: 10.10.10.12

(Desktop → IP Configuration → Static)

Top LAN (PC0) — 10.10.40.0/24

Gateway: 10.10.40.1
Mask: 255.255.255.0

- PC0: 10.10.40.10

Bottom VLAN20 (Switch2 PCs) — 10.10.20.0/24

Gateway: 10.10.20.1
Mask: 255.255.255.0

- PC7: 10.10.20.10

- PC8: 10.10.20.11

- PC9: 10.10.20.12

Bottom VLAN30 (Switch3 PCs) — 10.10.30.0/24

Gateway: 10.10.30.1
Mask: 255.255.255.0

- PC4: 10.10.30.10

- PC5: 10.10.30.11

- PC6: 10.10.30.12

Right DHCP LAN (Switch1 PCs) — DHCP

- PC1 / PC2 / PC3 → Desktop → IP Configuration → click DHCP

They should get 12.4.0.x automatically.

STEP 7 — Tests (must work if everything is correct)

A) Inter-VLAN test

From PC7:

ping 10.10.30.10

From PC4:

ping 10.10.20.10

ყველაფერი იმუშავებს ამის შემდეგ და ეგ არის.